✈️ N205EN — RV-10 COCKPIT CARD (PAGE 1)

Engine Leaning Schedule & Cockpit Rules of Thumb

1. AIRCRAFT & FUEL SYSTEM SPECS

| Fuel System: | 60.0 Gal Usable (2x 30.0 Gal L/R, 100LL, Single-Tank Feed) |
| --- | --- |
| Cruise Baselines: | Normal: 155 kt / 11.0 GPH ¦ Economy: 145 kt / 10.0 GPH |
| Climb Baseline: | 130 kt TAS / 17.0 GPH @ 500 ft/min (Safe Still-Air Range: ~670 NM) |

2. CLIMB ALTITUDE LEANING SCHEDULE (ALTIMETER MSL)

| Altimeter (MSL) | Target GPH | Avg EGT Target | Cyl #4 EGT | Cockpit Action |
| --- | --- | --- | --- | --- |
| Takeoff (SL) | 21.2 GPH | ~1,180°F | ~1,240°F | Full Rich for Takeoff |
| 1,100 - 2,000 ft | 17.0 GPH | 1,258°F | 1,326°F | Set Initial Lean at 1,000 AGL |
| 2,000 - 4,000 ft | 16.1 GPH | 1,268°F | 1,335°F | Lean ~1.0 GPH (~1 turn) |
| 4,000 - 6,000 ft | 15.0 GPH | 1,314°F | 1,368°F | Lean ~1.0 GPH (~1 turn) |
| 6,000 - 8,000 ft | 14.2 GPH | 1,320°F | 1,368°F | Lean ~0.8 GPH |
| 8,000 - 10,000 ft | 13.4 GPH | 1,314°F | 1,357°F | Lean ~0.8 GPH |
| 10,000 - 12,000 ft | 12.8 GPH | 1,287°F | 1,326°F | WOT / Fine Adjust |
| 12,000 - 14,000 ft | 12.2 GPH | 1,257°F | 1,288°F | Transition to Cruise |

3. OPERATIONAL RULES OF THUMB & LEANING

🎯 Pre-Set 1,000 AGL Target: Adjust mixture directly to ~1,260°F Avg EGT (~1,325°F Cyl #4) / 17.0 GPH. No peak hunting required.

📉 Altitude Leaning Schedule: Lean mixture by ~1.0 GPH (~1 full turn) per 1,000 ft of climb from Takeoff (~21 GPH) to TOC (~13 GPH).

🔥 EGT Sensitivity Factor: 1.0 GPH mixture lean yields ~33°F EGT rise. (If 50°F cold of target, lean knob ~1.5 GPH / 1.5 turns).

✈️ N205EN — RV-10 COCKPIT CARD (PAGE 2)

Flight Planning, Cruise Trade-Offs & Range Envelope

4. CRUISE PERFORMANCE MODES

| Mode | Cruise Speed | Fuel Flow | Efficiency | Context / Application |
| --- | --- | --- | --- | --- |
| Normal Cruise (Recommended) | 155 kt TAS | 11.0 GPH | 14.1 NM/gal | Standard cross-country (6k-10k ft, ~62% Power) |
| Economy Cruise | 145 kt TAS | 10.0 GPH | 14.5 NM/gal | Deep lean / low power cruise (~50-55% Power) |
| High Altitude Cruise | 163 kt TAS | 10.8 GPH | 15.1 NM/gal | Cruising > 10,500 ft MSL |

5. CRUISE MODE TRADE-OFF: 155@11 vs. 145@10

| Route Distance | 145 kt @ 10 GPH | 155 kt @ 11 GPH | Time Saved | Extra Fuel Burned |
| --- | --- | --- | --- | --- |
| 300 NM (e.g. Boston) | 2h 04m (20.7 gal) | 1h 56m (21.3 gal) | ⏱️ 8.0 min faster | +0.6 gallons |
| 500 NM (e.g. Atlanta) | 3h 27m (34.5 gal) | 3h 14m (35.5 gal) | ⏱️ 13.3 min faster | +1.0 gallon |
| 600 NM (e.g. Chicago / Jax) | 4h 08m (41.4 gal) | 3h 52m (42.6 gal) | ⏱️ 16.0 min faster | +1.2 gallons |

6. NON-STOP RANGE ENVELOPE FROM MILLVILLE (KMIV)

🧭 WEST: Chicago, IL (KMDW) — 596 NM | 3h 52m @ 155kt | 17.4 gal reserve remaining

🧭 SOUTH: Jacksonville, FL (KJAX) — 624 NM | 4h 02m @ 155kt | 15.6 gal reserve remaining

🧭 SOUTHWEST: Atlanta, GA (KPDK) — 553 NM | 3h 34m @ 155kt | 20.8 gal reserve remaining

🧭 NORTH: Montreal (CYUL) 371 NM / Toronto (CYYZ) 330 NM | ~2h 15m | ~33.0 gal reserve remaining